���������������������������������������������������������������������������Ŀ

� TRACKJOY - Gravis Ultrasound Tracker by RSC                               �

� (c) Copyright 1993, 1994, 1995 Tomi Joy                                   �

� All rights reserved                                                       �

� Programming & documentation: Tomi Joy                                     �

�                                                                           �

� TECHNICAL DOCUMENTATION for programmers                                   �

�                                                                           �

� *** NOTE: This is not supposed to be a particularly pretty                �

�           document, but is does supply some useful and                    �

�           possibly interesting information.                               �

�                                                                           �

� Specs are theoretically for Trackjoy version 1.00,                        �

� which is technically equivalent to all the                                �

� less than or equal to 1.00 beta versions.                                 �

� The file format specifications currently cover                            �

� *file* versions 1.0 and 1.1, the only difference being                    �

� in the way four bytes in then sample controllers are used.                �

�                                                                           �

�����������������������������������������������������������������������������

� Note: You might see some references to a program called Instrument

Maker in this document. IM is the sample editor which will be

included in future versions of Trackjoy. At the moment it doesn't

support 16-bit samples and has some severe bugs, so I didn't want to

release it yet.

� I don't promise that the information in this document is correct

and/or up-to-date.

�����������������������������������������������������������������������������

Table of contents

�����������������������������������������������������������������������������

1. Overview

2. The Gravis Ultrasoud

3. Trackjoy system requirements + other information

4. Trackjoy music storage & playing definitions

5. File formats (.TJS, .JOY, .BLK, samples)

6. Compression schemes

7. The interrupt interface

8. The sound effect interface

�����������������������������������������������������������������������������

1.  Overview

�����������������������������������������������������������������������������

The heart of Trackjoy is the playing engine. It's a relatively simple

group of functions, one of which is hooked to the timer interrupt vector

08h. This function increments counters and checks for the next notes to

play. It also checks when to damp voices down with the Ultrasound's

volume ramping register. This is done to avoid clicking when samples

are abruptly cut off. The damping timing can be adjusted to some extent.

The other functions associated with the playing engine simply fetch

the note, volume and instrument (and special command) data from the

storage array. One of the functions does the opposite of damping the

voices, ie. it lights them if the time is ripe.

Normally when an 80x86 is booted up, the timer interrupt vector points

to a routine which increments the time and date counters in the PC's

memory. The frequency at which this is done is approximately 18.2 Hz.

Unfortunately this gives rise to a problem: Trackjoy needs a 100 Hz

timer for the player engine, so the timer has to be re-programmed.

Obviously if we still keep on chaining back to the old service

routine after the player has done with an interrupt, time is going to

go too fast. If, on the other hand, we don't chain to the previous

vector at all, time is going to *stop*. Now we don't want that either.

The solution (although not a very pleasing one) is to call the old

routine only every now and then.

Trackjoy uses a 500 Hz hardware timer, but uses only every fifth tick

(100 Hz) to do anything to the music score.

Data bytes recognized by Trackjoy are the note, volume, instrument

bytes. On so-called FULL channels 3 more bytes are used: the command

and its two parameters. Notes are very straight forward: the value 0 is

C-0, 1 is C#0 and so on. When *any* of the data bytes has the falue 255

or FFh, it's an *empty* byte and is represented by dots on the Trackjoy

editing screen.

Semitones are supposed to be about 2^(1/12) * the previos note. ("The

twelth root of two" equals "two to the power of one twelth".) In

decimal this is about 1.05946. In other words, the frequency of any

note in an octave is basic_frequency * 1.05947 ^ (number_of_note). To

raise a note to an octave multiply it by 2 ^ (number_of_octave).

Actually, that is *not* exacty what Trackjoy does. It looks up notes

from a table, since it's much quicker *and* it avoids using floating

point math that way.

����������������������������������������������������������������������������

2. The Gravis Ultrasound

����������������������������������������������������������������������������

Let's be blunt: the Gravis Ultrasound card is a bugger to program.

Consider these "features" of the card from the programmer's point

of view.

1. 16-bit samples can't be played across 256k boundaries

(Memory is segmented as 1 to 4 256k banks)

2. With 32 voices in use, the maximum output frequency is only

19293 Hz, and 44.1 kHz is achieved only with 14 voices.

3. The frequency the card produces with a certain frequency

controller register value (and the speed at which volume ramping

is done) depends on the number of active voices in use.

4. Without significant programming effort, clicking is a real

problem. The click is caused by suddenly changing the DAC value by

a large degree. To remove it, you have to do a lot of careful

volume ramping register programming.

5. It only has 16 stereo pan positions. This isn't enough, because

the say in the official GUS documentation that it causes clicking.

6. Certain registers (the self-modifying ones) have to be updated

twice before you can be sure the card got the message.

�����������������������������������������������������������������������������

3. Trackjoy system requirements + other information

�����������������������������������������������������������������������������

Trackjoy was programmed in C and assembler

System requirements:

- 80286+

- EGA, although VGA is supported

- Ultrasound card (256k will do)

- about 300k free low memory

- hard disk stronly recommeneded

����������������������������������������������������������������������������

4. Trackjoy music storage & playing definitions

����������������������������������������������������������������������������

4.1 Music score is divided into patterns.

4.2 Pattern length (1-255 rows) and structure (ie. width and type

for each channel) can vary

4.3 Patterns consist of channels (max. 17 channels) Actual voice

channels are limited to sixteen in order to retain acceptable sound

quality with the GUS. In practice this will be quite enough for

almost any purpose. The 17th channel is an optional global channel

that controls all channels.

4.4 Three channel types are supported. They are:

Stripped: Only note, instrument & volume information

Full:     Note, instrument, volume & special effect info

Global:   Global volume & special effect information (max one per pattern)

(The CHORD type was omitted due to programming difficulties, not

actually needing it and most important of all � laziness.)

4.5 Notes: C-0 to B-9 (could in theory go to D-21, but that wouldn't be

practical)

4.6 Instruments: Normal 8- and  16-bit samples will be supported in version

1.0. Version 1.1 might add ADSR support to these samples. Sorry, no

GF1 patch support.

4.7 Channel sample volume: 0 - 254. This will be from a table, since

the US's volume 0-4095 is logarithmic instead of linearic.

4.8 Special effects (Full channels)

(Look for these in the next version)

4.10 Data requirements (in bytes per row per channel):

Stripped channels.... 3 bytes

Full channels........ 6 bytes

Global channels...... 4 bytes

If simple silence packing is implemented, this will be a lot less

in most instances. It probably won't, though, so no need to get

excited.

������������������������������������������������������������������������������

5. File format (Trackjoy saves and loads both songs and modules)

������������������������������������������������������������������������������

Trackjoy music file format version 2.0 (It *should* be impossible

to come across a version 1.X file, since this � the very first �

release of Trackjoy writes file version 2.0)

Format of song (.TJS) and module (.JOY)

��������������������������������������������������������������������

OFFSET:       PURPOSE:

�������       ������������������������������������������������������

0-7           8 bytes: "TRACKJOY"

8             1 byte: 0=song, 1=module

9             1 byte: file version, 20 decimal (20 means 2.0)

10            2 bytes (reserved, but Trackjoy writes 0x99 and 0x52)

12            1 word: default tempo

14            1 word: tempo modifier (not used)

16            1 word: master volume (max. 256)

18            1 word: volume modifier (not used)

20            1 byte: transpose

21            1 byte (reserved)

22            1 word: number of directory entries

Trackjoy can't write more than 196 (128 patterns,

64 samples + 4), and even this is more than anybody is

usually going to use.

24            Directory

[1 DWORD:PTR][1 BYTE:TAG][1 BYTE:UNUSED]

<number of dir entries> of these

The DWORD is a real byte-precision pointer to the object.

Objects will start at even byte offsets.

The first byte (tag) in a directory entry defines the

object pointed to.

Tags

0 � Writer information (text)

1 � Song name

2 � Song composer

3 � Song comment

4 � Pan positions

5 � Song order list

6 � Pattern

7 � Sample parameter block (no sample data)

8 � Sample parameter block + sample data

All objects in the directory are sorted in ascending

order by tag, ie., if song name exists, it will always

be before song composer etc.

The patterns come in the order in which they will be

in memory. The order of sample depends on the sample

slot (sample number) they go into, but Trackjoy writes

them in ascending order (by sample number).

Trackjoy will always write the order first, then all the

actual patterns and finally sample parameter blocks (with

or without samples). Files written by Trackjoy can be read

according to the directory almost sequentially to minimise

the necessity for random access. However, it is legal for

any of the objects to reside anywhere in the file.

Please remember that all objects start at even byte

addresses, so there will be some padding bytes around

in the file.

Note that each song and module has at least one pattern

and the order array. There may not be any instruments, and

the song information text strings are optional.

The only difference between a song and a module is

the byte at offset 8 from the file's beginning and

that in modules, the samples have tag 8 instead of 7.

Of course, the *format* allows both types of sample in

either song or module, but Trackjoy doesn't support this

at least at the moment.

Anything with a tag greater than 8 will promptly be

ignored by Trackjoy.

From here on, offsets will be relative to the beginning of the object in

question.

Format of writer information, song name, song composer and song comment:

0             1 word: length of string

2             data

Trackjoy will ignore parts of strings exceeding 80 bytes.

Trackjoy currently does not write "writer information"

at all.

Format of pan positions

0             1 word: length of string, Trackjoy currently writes 18

2             Byte array. The value 0 means left, 100 means right.

Format of sample parameter block

0             1 byte: sample number (the instrument number in patterns)

1             name: 30            (ASCIIZ)

filename: 13        (ASCIIZ)

type: 1             (0,1,2)

playmode: 1

allocated-flag: 1   (used internally by Trackjoy)

loop_beg: 4 (dw)

loop_end: 4 (dw)

length: 4 (dw)

gus_off: 4 (dw)     (used internally by Trackjoy)

freq: 2             (default 4000)

sampvolume: 2       (max. 256)

padding: 2  *** Not currently used ***

Some sample (slot) numbers may not appear at all, so just

leave the left-over slots empty.

The format of a "sample parameter block + sample data" is exactly

the same as "sample parameter block" except that is is followed

by exactly <length> bytes of sample data, whether 8- or 16-bit.

<Length> is found within the parameter block.

Format of pattern:

0             1 word: rows (Trackjoy's maximum is 256, and

the absolute maximum is 512)

2             1 byte: width in channels (for first pattern)

3             1 byte (reserved)

4             1 byte: compression scheme (0=None, 1=Silence)

5             33 bytes = format array (SEE NOTE BELOW)

38            1 word = actual length of written pattern data

(length of uncompressed final pattern is computed

from the format array, the width and the number

of rows)

40            value at [38] bytes of pattern data, compressed

or uncompressed

Format of order array:

0             1 word: length (always 128)

2             128 bytes: pattern play order array, FFh = empty

*** NOTE: Each byte of the format array indicates what type of channel is at

that index. The value 0 is for FULL channels, 1 for STRIPPED

and 2 for GLOBAL channels. There are 33 bytes because TRACKJOY

is built so that it can (by modifying a #define directive) be

made to operate on max. 33 channels.

Pattern compression:

Three byte values are reserved for special purposes. They are,

in decimal, 233, 231 and 237. These values were chosen because

they are bigger than valid note, instrument or special command

values and are fairly unlikely to be entered as volumes by the

user.

Byte/byte pair  Interpretation

231             Repeat FFh twice

233             Repeat FFh three times

237,0           231

237,1           233

237,2           237

237,n when n>2  Repeat FFh n+1 times

In other words, 231 and 233 are always alone. 237 is always

followed by another byte. The actual values 231 and 233 are

encoded through 237. This ensures that the worst compression

ratio when compressing strips of FFh longer than 1 byte will

be 1:2. The best is 1:128. FFh is by far the most common

byte in Trackjoy patterns. It is not worth bothering about

other values, as the extra_compression / annoyance_of_

extra_programming factor is very low indeed, not to mention

that happiness_of_Tomi = 1 / Time_spent_in_extra_programming.

Note that pattern data is compressed COLUMN BY COLUMN,

not row by row. This yields significantly better compression ratios

than row-by-row compression, since the way data is entered into

a pattern is more uniform vertically than horizontally.

Example pattern:

Let X be the width (in 1-byte parameter columns like <note>,

<ins> or <special command>) of the current pattern, and

let there be 3 rows in our pattern.

�������������������� � �

0+ �0�1�2�3�4�5�6�7�...

�������������������� � �

X+ �A�B�C�D�E�F�G�H�...

�������������������� � �

2X+ �L�M�N�O�P�Q�R�S�...

�������������������� � �

In this pattern, you would compress or uncompress in order

0,A,L,1,B,M,2,C,N,3... What type of field you are compressing doesn't

matter, but knowing the pattern width is important. Note that

compression from column to column (for instance, from P to 5) is

allowed. This compression scheme will of course only allow compression

of strips max. 256 times FFh in length, at a time.

Uncompression example:

The string

<16><12><255><12><255><12><233><13><237><3><237><1>

uncompresses to

<16><12><255><12><255><12><255><255><255><13>

<255><255><255><255><233>.

For obvious reasons, things like

<231><231>

<231><233>

<231><237><64>

<233><255>

should never appear in the middle of a column. However, they are

perfectly correct and readable, and may occur at the very last column

of a pattern. Within a column, there are of course more logical ways of

compressing these oddities. However, <237><255> followed by <255>,

<231>, <233> or another <237><n> may appear anywhere if 257, 258 or more

FFh's are compressed.

For technical reasons, Trackjoy actually rotates and flips a pattern's

data before compression, ie. a pattern is treated as a rectangular

byte block with a width of <columns> and height of <rows>.

Ie.,

abcd              aei

efgh    becomes   bfj   which is like a 90� degree rotate clockwise

ijkl              cgk   and a flip horizontal.

dhl

After the 90 degree rotate (and flip) a normal run-length crunching

algorithm is used, and the output is saved to disk with the correct

parameters. This is the easiest way to do it :)

5.2 Trackjoy block file format

Format of block file (.BLK)

��������������������������������������������������������������������

OFFSET:       PURPOSE:

�������       ������������������������������������������������������

0             11 bytes: "�R�R�R�R�R!"

To understand why these first eleven bytes are

"�R�R�R�R�R!", you really have to be an insider. Sorry

about that. (According to some not-very-reliable sources,

"�R�R�R�R�R!", derived from the verb "�rist�", is the

closest ASCII equivalent of the the sound emitted from a

Class A Finnish "puliukko" on discovering that he has � in

one way or an other � lost contact with his best friend �

the bottle of "Kossu". The puliukko is a species of

red-nosed scraggy-looking being completely pickled in

alcohol and found in abundance in the parks of Helsinki.

�r�r�r�r�r�r�r!)

11            word: left

13            word: top

15            word: right

17            word: bottom

19            word: length

21            block data in linear uncompressed full channel format

(ie., channel 1 row 0 is right after channel 0 row 0)

channel 0 row 0) END

5.3 Sample formats

5.3.1 Raw 8-bit PC8

This is the unsigned typical PC sample type with no header.

Each sample is simply a value from 0 to 255, and +128 represents

an output voltage of zero. It needs to be converted to signed

format for most purposes including volume scaling and playing

with the Ultrasound.

5.3.2 Signed raw 8-bit A8

This is the same as PC8 except that it is in signed two's

complement format. The values are signed integers from

-128 to +127, zero naturally represents an output volume

of zero.

5.3.3 Unsigned raw 16-bit S16

This is the 16-bit format Trackjoy currently reads. Samples are unsigned

16-bit integers (words) with 32768 representing the output voltage of

zero. Also this format is converted to signed 16-bit (by flipping bit

15) before loading into the Ultrasound's memory. The byte order is

low-high, ie. the least significant 8 bits of the value occupy the lower

address. This is the Intel back-words integer storage format.

5.3.4 Trackjoy headered sample format

Format of TJINSX headered sample, version 1.1

��������������������������������������������������������������������

OFFSET:       PURPOSE:

�������       ������������������������������������������������������

0             5 bytes: "TJINS"

5             1 byte: version number, 0Bh (11 decimal = 1.1)

*** Ignore any files with this byte less than 0Bh.

6             16 bytes: "XXXXXXXXXXXXXXXX", reserved for future

use, of course

22            sample information, precisely same format

as in songs and modules (See song & module formats,

offset x+1 or definition of "sampleinfo" structure)

90            sample data in signed format, low-high byte order

for 16-bit samples, can be dumped directly into

the Ultrasound and then played

END

5.3.5 Scream Tracker III / Digiplayer sample format

Trackjoy will read the basic Scream Tracker III sample format (8-bit

mono), and Instrument maker will, in addition to reading these files,

also write them. For a definition of the Scream Tracker III / Digiplayer

sample format, consult the TECH.DOC file included in the public release

package of Scream Tracker III.

�����������������������������������������������������������������������������

7. The interrupt interface

�����������������������������������������������������������������������������

Trackjoy supports an interrupt interface for other programs to control

music (or sound effect) output. The interface can also be used for

synchronising other programs with the player engine. TRACKJOY can

hook itself to one of the interrupts between 60H � 66H, namely int 61h.

When the interface (int 61h) is called, AL contains the function

number. AL doesn't return a value. (W) means AH and BX contain

information to pass to TRACKJOY (functions 0h - Fh). (R) means another

program reads information from TRACKJOY, which returns it in the other

registers (AH, BX, CX and in a few cases, DX). Pointers and long ints

are dwords and are returned in the register pair BX:CX.

The interrupt can be called either from an assembly language program by

simply setting the registers to the desired values or through one of

the interrupt interfaces supplied with your high-level compiler.

(Syntax for popular C-compilers for MS-DOS offer the following

interface:

#include <dos.h>

...

call_trackjoy(void)

{

union REGS inregs, outregs;

unsigned interrupt_number = 0x61;

inregs.h.al = 0x10; /* call "READ MUSIC CONTROL" */

int86(interrupt_number, &inregs, &outregs);

/* Data returned is in outregs.h.ah, outregs.h.bl,

outregs.h.cl and outregs.h.ch, in this case. */

...

}

...)

These are the functions that are supported:

==============================================================================

WRITE COMMANDS 00h - 09h, 20h - 2Fh

==============================================================================

AL==00h  (W)  MUSIC CONTROL

AH = Select function: 0 = none, 1 = play/stop, 2=pause/play

1: play/stop - If BL == 0 TRACKJOY will stop

2: play/pause - If BL == 0 TRACKJOY will deep-freeze.

Ie., the timer interrupt is un-hooked from TRACKJOY,

the timer is set to 18.2 Hz and all the GUS accumulators

are frozen. This is equivalent to a pause state.

BL == 1 melts TRACKJOY...

------------------------------------------------------------------------------

AL==01h  (W)  MASTER VOLUME

AH = new master volume (0-255)

------------------------------------------------------------------------------

AL==02h  (W)  CHANNEL CONTROL

AH = number of channel

if CL == 1 TRACKJOY will mute channel <AH>

------------------------------------------------------------------------------

AL==03h  (W)  TRANSPOSE

AH = notes to transpose (if bit 7 set, transpose down)

------------------------------------------------------------------------------

AL==04h  (W)  PITCH BEND

new_global_pitch = global_pitch * BX / 1000;

------------------------------------------------------------------------------

AL==05h  (W) SET PITCH

BX = global pitch

------------------------------------------------------------------------------

AL==06h  (W) SET GLOBAL TEMPO

BX = new global tempo

------------------------------------------------------------------------------

AL==07h  (W) FORCE TRACKJOY TO PLAY A PATTERN

AH = pattern to play (0 = first)

------------------------------------------------------------------------------

AL==08h  (W) JUMP ROW

AH = row to jump to in current pattern (0 = first)

------------------------------------------------------------------------------

AL==09h  (W) END "P" COMMAND POLLING

If TRACKJOY is polling for keyboard input (invoked by the

"P" pattern command), issuing this command will make TRACKJOY

continue to play the current pattern.

*** NOTE: This command is read from AL==1Eh, *NOT* AL==19h!

------------------------------------------------------------------------------

AL==20h  (W) PLAY NOTE

AH = number of sample

BL = note

BH = GUS channel to play through

==============================================================================

READ COMMANDS 10h - 1Fh, 30h

==============================================================================

AL==10h  (R)  MUSIC CONTROL

If AH bit 0 is OFF, music is stopped

If TRACKJOY is running, BL == 'T' (54h) and  BH == 'J' (4Ah)

CL = minor version number

CH = major version number

If DL bit 0 is ON, Trackjoy is paused

------------------------------------------------------------------------------

AL==11h  (R)  MASTER VOLUME

AH = master volume

------------------------------------------------------------------------------

AL==12h  (R)  CHANNEL CONTROL

(w) AH = number of channel

BL = 1 for a short period, if a note was turned on

BH = state of voice (1 = GUS is playing voice, 0 = stopped)

CL = state of channel (1 means muted)

------------------------------------------------------------------------------

AL==13h  (R)  TRANSPOSE

AH = semitones transposed (if bit 7 set, transposed down)

------------------------------------------------------------------------------

AL==14h  (RESERVED, Pitch Bend can't be read. Use AL==15h instead)

------------------------------------------------------------------------------

AL==15h  (R) READ PITCH

BX = global pitch

------------------------------------------------------------------------------

AL==16h  (R) READ GLOBAL TEMPO

BX = global tempo

------------------------------------------------------------------------------

AL==17h  (R) READ CURRENT PATTERN AND ORDER

AH = current pattern (0 = first)

BX:CX points to order string (128 bytes)

DL = current order

------------------------------------------------------------------------------

AL==18h  (R) READ CURRENT ROW

AH = current row in pattern (0 = first)

BL = rows in current pattern

------------------------------------------------------------------------------

AL==19h  (R) GET "sampleinfo" STRUCTURE ARRAY ADDRESS

struct sampleinfo sample[SAMPLES];

BX:CX equals &sample[0]

------------------------------------------------------------------------------

AL==1Ah  (R) GET "gdram" STRUCTURE ADDRESS

struct gus_dram gdram;

BX:CX equals &gdram

------------------------------------------------------------------------------

AL==1Bh  (R) GET NAME ASCIIZ ADDRESS

char name[80];

BX:CX equals &name

------------------------------------------------------------------------------

AL==1Ch  (R) GET COMPOSER ASCIIZ ADDRESS

char composer[80];

BX:CX equals &composer

------------------------------------------------------------------------------

AL==1Dh  (R) GET COMMENT ASCIIZ ADDRESS

char comment[80];

BX:CX equals &comment

------------------------------------------------------------------------------

AL==1Eh  (R) CHECK PATTERN POLLING STATE

If AH==1, TRACKJOY is waiting for a interrupt with AL==09h

to continue with the current pattern. This state is invoked

with the "P" pattern command.

*** NOTE: This command is written to AL==09h, *NOT* AL=0Eh!

------------------------------------------------------------------------------

AL==1Fh   (R) READ ERROR LOG

If AH != 0, TRACKJOY has an error to report

BX:CX points to the error message string

------------------------------------------------------------------------------

AL==30h   (R) READ TIMER

BX:CX = (unsigned long) timer ticks from start of play

DX = (unsigned int) timer frequency

------------------------------------------------------------------------------

This is the format of the SAMPLEINFO structure. It controls the audio

samples TRACKJOY uses as instruments.

*** NOTE: The constant SAMPLES = 64.

struct sampleinfo {      /* sample information */

char name[30];   /* sample description */

char file[13];   /* sample filename */

char type;       /* file type (16/PC8/A8) flag */

char playmode;   /* this is what's actually squirted into the GUS */

char acd;        /* allocated flag */

long loopbeg;    /* loop beginning */

long loopend;    /* loop end */

long length;     /* sample length (bytes) */

long gusoff;     /* offset in GUS memory */

int freq;        /* sample frequency */

unsigned vol;    /* sample volume */

unsigned pad;    /* padding */

};

Storage definition in TRACKJOY.C:

struct sampleinfo sample[SAMPLES];

This is the format of the GUS_DRAM structure:

struct gus_dram {                       /* memory control block */

unsigned long used;             /* how much memory is allocated */

unsigned long beg[SAMPLES];     /* beginning of a block */

unsigned long length[SAMPLES];  /* length of a block */

char stat[SAMPLES];             /* status. 0 = empty, 1 = allocated,

2 = free */

int c;                          /* number of blocks */

};

*** NOTE:  Status 0 (empty) always means the free hunk of memory from

the end of the last allocated block to the end of memory.

Status 1 means the block in use, and status 2 means the

block has been freed, and can be re-used. Status 2 blocks

only exist before allocated blocks of memory. (A Status 2

block at the end of allocated memory is always converted to

status 0 when the block is freed.)

Storage definition in TRACKJOY.C:

struct gus_dram gdram[4];

Each gdram in the above array is for a 256k Ultrasound memory bank.